COMBINE HARVESTER TEST FORM

DECLARATION REGARDING SELECTED PERFORMANCE AND OTHER CHARACTERISTICS AS PER IS: 15806-2008

| Sr. No. |  | Characteristics | Requirement | Declared |
| --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 |
|  | Prime Mover Performance: |  |  |  |
|  | i) | Max. power (absolute) Average max. power observed during 2 hrs. max. power test in natural ambient condition kW | It should not be less than 5% of the declared valve. |  |
|  | ii) | Max. power observed during test after adjusting the no load engine speed as per recommendation of the manufacturer for field work, kw | Max. Power observed must not be less than 5% of declared value. |  |
|  | iii) | Power at rated engine speed, kw | The observed value must not be less than 5% of the declared value by the applicant. |  |
|  | iv) | Specific fuel consumption g/ kwh. | The average value during 2 hr. max. Power test must be within ±5% of the declared value by applicant/manufacture. |  |
|  | v) | Max. smoke density (bosch no.) at 80% load between the speed at max. Power & 55% of speed at max. Or 1000rpm whichever is higher should be observed as per CMVR rule. | For tractor:- 5.2 bosh no. or 75 hartridge For engine:- Free deceleration or natural aspirated or turbo charges 65 hartridge |  |
|  | vi ) | Max. crank shaft torque, (N-m) observed during the test after no load engine speed is adjusted as per manufacture’s recommendation for field work | It must not be less than 8% of declare value of manufacturer. |  |
|  | vii) | Back up torque, % | 7% min. |  |

| 1 | 2 |  | 3 |  | 4 |  |  | 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | viii) |  | Max. operating temp. to be declared by manufacturer |  | - engine oil - coolant |  |  |  |
|  | ix) |  | Lubrication oil consumption, g/kwh |  | 1% of SFC at max. Power during high ambient condition. |  |  |  |
|  | Brake Performance: |  |  |  |  |  |  |  |
|  | i) |  | Max. stopping distance at a force equal to or less than 600 N on break pedal, m |  | 10 m or s≤0.15v +v²/130 v=speed corresponding to 80% of design max. Speed, kmph. |  |  |  |
|  | ii) |  | Max. force exerted on brake pedal to achieve a deceleration of 2.5 m/sec² |  | ≤ 600N. |  |  |  |
|  | iii) |  | Whether parking brake is effective at a force of 600 N at foot pedal or 400 N at hand and lever |  | Yes or No. |  |  |  |
|  | Mechanical Vibration: |  |  |  |  |  |  |  |
|  | i) |  | Operator’s platform |  | 120µm max. |  |  |  |
|  | ii) |  | Steering wheel |  | 150µm max. |  |  |  |
|  | iii) |  | Seat with driver seated |  | 120µm max. |  |  |  |
|  | Air cleaner oil pull over(%) |  |  |  | 0.25 |  |  |  |
|  | Noise Measurement : |  |  |  |  |  |  |  |
|  | i) |  | Max. ambient noise emitted by combine db(A) |  | 88 dB (A) as per CMVR |  |  |  |
|  | ii) |  | Max. noise at operator’s ear level db(B) |  | 98 dB (A) as per CMVR. |  |  |  |
|  |  | Discard Limit: |  |  |  |  |  |  |
|  |  | i) |  | Cylinder bore diameter |  | Should not exceed the values declared by the manufacture |  |  |
|  |  | ii) |  | Piton diameter |  | -do- |  |  |
|  |  | iii) |  | Clearance between piston and cylinder liner at skirt |  | -do- |  |  |
| 1 |  | 2 |  | 3 |  | 4 |  |  |
|  |  | iv) |  | Ring End gap Top comp. ring. 2nd comp. ring. Oil ring |  | -do- |  |  |
|  |  | v) |  | Ring groove clearance Top comp. ring. 2nd comp. ring. Oil ring |  | -do- |  |  |
|  |  | vi) |  | Diametrical clearance of main bearing |  | -do- |  |  |
|  |  | vii) |  | Crank shaft end float |  | -do- |  |  |
|  |  | viii) |  | Diametrical clearance of big end bearings |  | -do- |  |  |
|  |  | ix) |  | Axial clearance of big end bearings |  | -do- |  |  |
|  |  | x) |  | Thickness of brake lining |  | -do- |  |  |
|  |  | xi) |  | Thickness of clutch plate |  | -do- |  |  |
|  |  | Field Performance: |  |  |  |  |  |  |
|  |  | i) |  | Suitability for crops |  | Paddy |  |  |
|  |  | ii) |  | Grain breakage in grain tank |  | ≤ 2.5% |  |  |
|  |  | iii) |  | Non collectable |  | ≤ 2.5% for wheat, paddy & gram ≤ 4.0% for soybean |  |  |
|  |  | iv) |  | Threshing efficiency |  | ≥ 98% wheat & paddy |  |  |
|  |  | v) |  | Cleaning efficiency |  | ≥ 96% wheat & paddy |  |  |
|  |  | Safety requirement: |  |  |  |  |  |  |
|  |  | i) |  | Guards against all moving parts |  | Essential |  |  |

| 1 | 2 | 3 |  | 4 | 5 |
| --- | --- | --- | --- | --- | --- |
|  | ii) | Lighting arrangement - Head light - Parking light - Indication - Reverse gear - Brake - Number plate |  | Essential as per CMVR |  |
|  | iii) | Grain tank cover |  | Essential |  |
|  | iv) | Spark arrester in engine’s exhaust |  | Essential |  |
|  | v) | Stone trap before concave |  | Essential |  |
|  | vi) | Rear view mirror |  | Essential |  |
|  | vii) | Slip clutch at following driver – - Cutting platform - Under shout conveyor drive - Grain & tailing elevator |  | Essential |  |
|  | viii) | Anti slip surfaces at operator platform & ladder & proper gripping for the control levers. |  | Essential |  |
|  | ix) | Working clearance around the controls |  | Essential 70 mm, min |  |
|  | x) | Labeling of control, gauge |  | Essential |  |
|  | i) |  | Guard should conforms to IS: 6024- 1983 | The guard (except ledger plate) shall be manufactured from malleable iron casting (Is: 2108-1977), steel casting (Is: 1030-1947) or steel forging (IS: 2004-1978) |  |
|  | ii) |  | Knife blade As per IS: 6025-1982 | It must have chemical composition as C= 0.70-0.95% Mn= 0.30-0.50% |  |
|  | iii) |  | Knife back Must meet the requirement of IS: 10378-1982 | The knife back shall be manufactured from carbon steel having minimum carbon content of 0.35% |  |

| 1 |  |  | 2 | 3 | 4 |  | 5 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Labelling of combine harvester (Provision of Labeling plate): |  |  |  |  |
|  | 1) |  | Make |  | Should conform to the requirements of IS: 10273- 1987 along-with declared value of HP | -- |
|  | 2) |  | Model |  |  | -- |
|  | 3) |  | Year of manufacture |  |  | -- |
|  | 4) |  | Engine number |  |  | -- |
|  | 5) |  | Chassis number |  |  | -- |
|  | 6) |  | Declaration of power, (kW) |  |  | -- |